AIDS MICROPROCESSOR LAB S21 BATCH (2023-24)

Experiment 1 Title: Assembly language programming for MOV instruction in various addressing modes using software tool TASM 1.4

Name of student: Meet Raut        Class Roll Number: 2201084

Date of Performance: 11/03/2024

Batch: S2-1                    Timing: 3:00-5:00           Date of Submission: 05/02/2024

Assembly language code

data segment                  # Data segment

n1 db 15h                    # variable n1 define 8 bit number

n2 dw 1367h                 # variable n2 define 16 bit number

n3 dw 0                      #Initialize n3 to 0

n4 dw 0                      #unused variable

arr1 db 00h,12h,23h,34h,

45h,56h,67h,78h,89h,92h               #Array arr1

arr2 db 20h,21h,22h,23h,

24h,25h,26h,27h,28h,29h                #Array arr2

arr3 db 10 DUP(0)                      #Array arr3 with 10 elements initialized to 0

.

data ends                      # Assemble directives

code segment

assume cs:code, ds:data       # Assemble directives defining cs and ds

start:                                     # start of code segment

mov ax,data                         # moving data to ax register- initialization process

mov ds,ax                            # moving ax to dx register -- initialization process

;IMMEDIATE ADDRESING MODE

mov al,34h                             # Load immediate value 34h to al register

mov cx,1257h                            # Load immediate value 1257h to cx register

;REGISTER ADDRESSING MODE

mov ah,al                           # Copy the value from al to ah register

mov dx,cx                      # Copy the value from cx to dx register

;DIRECT ADDRESSING MODE

mov al,n1                #Load the value of variable n1 into al register

mov bx,n2               #Load the value of variable n2 into bx register

mov n3,al                #Copy the value of al register to variable n3

mov bx,OFFSET arr1    #Load the offset address arr1 into bx register

mov si,OFFSET arr2     #Load the offset address arr2 into si register

mov di,OFFSET arr3   #Load the offset address arr3 into di register

;INDIRECT ADDRESSING MODE

mov cl,[bx]            #Load the value at the address stored in bx into cl register

mov ch,[si]          #Load the value at the address stored in si into ch register

mov [di],ch       #Store the value of ch in the address stored in di

;BASE ADDRESSING MODE

mov dl,3[bx]          #Load the value at the address (bx + 3) into dl register

;INDEXED ADDRESSING MODE

mov 7[di],dl       #Store the value of dl at the address (di + 7)

mov ah,4ch        #Set ah register for exit code

int 21h                                 # breakpoint interrupt

code ends                       # Assembler directives to end code

end start                        # Assembler directives to end start

Result :

(i) IMMEDIATE ADDRESSING MODE:

(ii) REGISTER ADDRESSING MODE:

(iii) DIRECT ADDRESSING MODE:

CONCLUSION: LO 1, LO 3 mapped.

---------------------------------*****************************-----------------------------------