AIDS MICROPROCESSOR LAB S21 BATCH (2023-24)

Experiment 3(a) Title: Assembly language programming to find minimum number from 10  8-bit hexadecimal numbers (GIVEN ARRAY) using software tool TASM 1.4

Name of student: Meet Raut        Class Roll Number: 2201084

Date of Performance: 26/02/2024

Batch: S2-1                    Timing: 3:00-5:00           Date of Submission: 26/02/2024

Assembly language code

data segment                               # Data segment

array db 04h,02h,03h,0Ch,05h,         # Declares an array named array with ten

07h,08h,0Ah,0Bh,01h                             8-bit hexadecimal numbers.

smaller db 00h           # Declares a variable smaller with an initial value of 00h.

data ends                      # Assemble directives

code segment

assume cs:code, ds:data       # Assemble directives defining cs and ds

start:                                     # start of code segment

mov ax,data                         # moving data to ax register- initialization process

mov ds,ax                            # moving ax to dx register -- initialization process

mov si,offset array            # Uses si register to point to beginning of the array.

mov cl,0Ah                          #Initializes a counter (cx) with the value 0Ah

mov al,[si]                        # moves the value at the array location to al.

inc si                                # Increments the si register

back: cmp al,[si]          # Enters a loop (back) that compares the value in al with  .                                       the value at the memory location pointed to by si

jnc skip               # If the value in al is not less than the value in the array .   .                                      (jnc skip), it skips to the label skip

skip: mov al,[si]            # moves the value at the array location to al.

inc si                   # Increments the si register

loop back           # repeats the loop until cx becomes zero.

mov smaller,al              # Stores the final value in al into the smaller variable.

int 03h                                # breakpoint interrupt

code ends                       # Assembler directives to end code

end start                        # Assembler directives to end start

Result:

Experiment 3(b) Title: Assembly language programming to find maximum number from 10  8-bit hexadecimal numbers (GIVEN ARRAY)  using software tool TASM 1.4

Name of student: Meet Raut        Class Roll Number: 2201084

Date of Performance: 26/02/2024

Batch: S2-1                    Timing: 3:00-5:00           Date of Submission: 26/02/2024

Assembly language code

data segment                               # Data segment

array db 01h,02h,03h,04h,05h,         # Declares an array named array with ten

07h,08h,0Ah,0Bh,0Ch                             8-bit hexadecimal numbers.

larger db 00h              # Declares a variable larger with an initial value of 00h.

data ends                      # Assemble directives

code segment

assume cs:code, ds:data       # Assemble directives defining cs and ds

start:                                     # start of code segment

mov ax,data                         # moving data to ax register- initialization process

mov ds,ax                            # moving ax to dx register -- initialization process

mov cx,0Ah                          #Initializes a counter (cx) with the value 0Ah

mov si,offset array            # Uses si register to point to beginning of the array.

mov al,00h                           # Initializes the al register with the value 00h.

back: cmp al,[si]          # Enters a loop (back) that compares the value in al with  .                                       the value at the memory location pointed to by si

jnc skip               # If the value in al is not less than the value in the array .   .                                      (jnc skip), it skips to the label skip

mov al,[si]            # moves the value at the array location to al.

skip: inc si                   # Increments the si register

loop back           # repeats the loop until cx becomes zero.

mov larger,al              # Stores the final value in al into the larger variable.

int 03h                                # breakpoint interrupt

code ends                       # Assembler directives to end code

end start                        # Assembler directives to end start

Result:

CONCLUSION: LO 2, LO 3 mapped.

---------------------------------*****************************-----------------------------------